Controller Requirement Specification Table

控制系統需求規格表

This document defines the information required from the client before vendors can create an accurate development plan and quotation.
本文件列出供應商在制定精確的開發計畫與報價前，必須由客戶提供的詳細資訊。

1. System Architecture & Control Framework

系統架構與控制框架

| Item 項目 | Description / Question 說明與需求 | Client Input (English) | 客戶填寫 (中文) |
| --- | --- | --- | --- |
| Control Platform 控制平台 | LinuxCNC? Version? Custom build? 是否使用 LinuxCNC？版本？是否自行編譯？ | LinuxCNC | 使用 LinuxCNC |
| API / Communication Protocol API／通訊協定 | HTTPS? REST? Python interface? Must follow existing Skatescribe structure? 是否沿用現行 HTTPS API 結構？ | Machine operations (running gcode, recovering from estop) are done via HTTP requests from Skatescribe app to LinuxCNC server. GitHub repo & API documentation will be shared. | 機台操作（執行 gcode、急停復原）透過 Skatescribe 應用程式以 HTTP 請求連接至 LinuxCNC 伺服器。將提供 GitHub 原始碼庫與 API 文件。 |
| Expected Control Stack 預期控制堆疊 | LinuxCNC ↔ Python ↔ Skatescribe App communication structure | LinuxCNC computer (server hosting TBD) & Windows computer (hosting Skatescribe app) connected via Ethernet. SMB connection required. | LinuxCNC 電腦（伺服器待確認）與 Windows 電腦（執行 Skatescribe 應用程式）透過乙太網路連接，需建立 SMB 連線。 |
| G-code Handling G-code 處理方式 | Sequence control? Custom M-codes? Required safety interlocks? | Operations: profiling/sharpening, scanning, reset table, warm-up (all axes). No custom M-codes. Standard safety checks. | 操作流程：修磨／銑削、掃描、重置工作台、全軸暖機。無自訂 M-code，採用標準安全檢查。 |

2. Required Machine Control Functions

機台核心控制功能需求

| Function 功能 | Description 說明 | Client Input (English) | 客戶填寫 (中文) |
| --- | --- | --- | --- |
| Assign Tool-to-Spindle 指派刀具至主軸 | Python command to assign selected tool | Yes | 是 |
| Store Tool into ATC 工具放回刀庫 | Specify ATC type (carousel / linear / custom) | Yes | 是 |
| Remove Tool from Spindle 主軸換刀 | Define tool-release logic | Yes | 是 |
| Tool Length Measurement 刀長量測 | Method? Touch probe? Fixed gauge? Laser? | Yes | 是 |
| Door Lock/Unlock 安全門鎖控制 | Interlock logic / override conditions | Yes | 是 |
| USB IO Relay Control | List relay-controlled subsystems | Yes | 是 |
| Emergency Shutdown 緊急停機 | Required conditions and triggers | Yes | 是 |
| Recover from ESTOP | Reset state to ON, home all axes | Yes | 是 |
| Execute GCode from file | Load and run nc/ngc file, pause/resume | Yes | 是 |
| Status polling 狀態輪詢 | Get machine states | Yes | 是 |

3. Machine Workflow Logic

機台作業流程邏輯

| Workflow 流程 | Steps Required 需要的步驟 | Client Input (English) | 客戶填寫 (中文) |
| --- | --- | --- | --- |
| Startup Procedure 開機流程 | Motion enable, homing, safety check | Bring task_state to STATE_ON, home all axes, run warm-up program. | 將 task_state 設為 STATE_ON，歸零所有軸，執行暖機程式。 |
| Tool Change Flow 換刀流程 | Retrieval → engagement → confirmation | No confirmation step in app. Prefer sensors for tool pickup/storage (optional). | 應用程式無確認步驟。建議使用感測器檢查刀具拾取／存放（非必須）。 |
| ATC Interaction ATC 動作邏輯 | Slot indexing, sensors, error handling | Linear ATC preferred. Sensor to confirm slot occupancy. | 傾向使用線性刀庫。建議加裝感測器確認刀位是否有刀具。 |
| Blade Measurement Flow 檢測流程 | Laser scanning steps / trigger logic | USB IO triggers batch start in Keyence controller. | 透過 USB IO 觸發 Keyence 控制器批次啟動。 |
| Sharpening/Profiling Cutting Flow 切削流程 | Feed, pass count, cutter mapping | Minimize material removal per pass. | 每次切削盡量減少材料去除量。 |
| Shutdown Flow 關機流程 | Safe stop → axis park → power-down | Correct as specified. | 與所列流程一致。 |

4. Required API Commands

必要 API 指令與回傳資料結構

| Command 指令 | Purpose 功能 | Request Structure 請求格式 | Response Structure 回傳格式 |
| --- | --- | --- | --- |
| /v2/status GET | Status ping 狀態檢查 | { } | { "status":"Success", "data":{...} } |
| /v2/spindle/tool GET | Get tool in spindle 取得主軸刀具 | { } | { status:"Success", data:{tool_in_spindle:2} } |
| /v2/door-switch/unlock POST | Unlock door 開啟門鎖 | { } | { status:"Success" } |
| /v2/door-switch POST | Update door lock status 更新門鎖狀態 | { enable_enclosure_door_switch:True } | { status:"Success" } |
| /v2/atc/slot/open GET | Check empty slot 檢查刀庫空位 | { } | { status:"Success", data:{is_full:True} } |
| /v2/atc/slot/register POST | Assign tool to slot 指派刀具至刀庫 | { tool_num:1 } | { status:"Success" } |
| /v2/atc/slot/register-current POST | Register spindle tool 登錄主軸刀具 | { tool_num:1 } | { status:"Success" } |
| /v2/atc/slot/store-current POST | Store spindle tool 存放主軸刀具 | { } | { status:"Success" } |
| /v2/atc/slot/tool POST | Remove tool 移除刀具 | { tool_num:2 } | { status:"Success" } |
| /v2/tool/measure-length POST | Measure tool length 測量刀長 | { } | { status:"Success", data:{length:12.445} } |
| /v2/tool/change POST | Assign tool to spindle 指派刀具至主軸 | { tool_num:1 } | { status:"Success" } |
| /v2/machine/reset POST | Recover from ESTOP 從急停復原 | { } | { status:"Success" } |
| /v2/machine/shutdown POST | Shutdown machine 關機 | { } | { status:"Success" } |
| /v2/program/run POST | Run gcode file 執行程式 | { file_name:scan.nc, wait:True } | { status:"Success" } |
| /v2/program/pause POST | Pause program 暫停程式 | { } | { status:"Success" } |
| /v2/program/resume POST | Resume program 繼續程式 | { } | { status:"Success" } |
| /v2/program/stop POST | Stop program 停止程式 | { } | { status:"Success" } |

5. Vision / Scanner Integration (Gen2 not required)

視覺／掃描模組需求（Gen2 不需）

| Item 項目 | Description 說明 | Client Input (English) | 客戶填寫 (中文) |
| --- | --- | --- | --- |
| Scanner Model 掃描器型號 | Current: Keyence LJ-V7080 | Not Needed | 不需要 |
| Trigger Logic 觸發邏輯 | How scanning starts/ends | Starting the scan is controlled by USB IO. | 掃描由 USB IO 控制啟動。 |
| Data Format 資料格式 | Raw profile? Filtered? Sampling rate? | Not Needed | 不需要 |
| Communication 介面 | TCP? Serial? UDP? | Not Needed | 不需要 |

6. Safety

You’re absolutely right — let’s finish the Safety & Interlocks section properly and then close out with the Additional Constraints / Requirements so the document is fully complete in the bilingual four‑column format.

Here’s the continuation and completion:

6. Safety & Interlocks

安全與聯鎖需求

| Safety Function 安全功能 | Requirement 需求說明 | Client Input (English) | 客戶填寫 (中文) |
| --- | --- | --- | --- |
| Door Interlock 門鎖 | Locked during spindle motion? | Locked during any motion | 在任何運動時鎖定 |
| E-stop 緊急停止 | Required states and reset logic | Fault all axes, machine state = STATE_ESTOP | 所有軸故障，機台狀態 = STATE_ESTOP |
| Power Loss Recovery 電源中斷復原 | Required steps? | Auto restart controller after power returns, machine remains estopped | 電源恢復後自動重啟控制器，但機台保持急停狀態 |
| Overtravel / Limit Switches 超程／限位開關 | LinuxCNC or hardware-level? | Hardware-level preferred (operators not CNC machinists) | 建議使用硬體層級（操作員非 CNC 技師） |

7. Additional Constraints or Requirements

其他額外需求

| Category 類別 | Requirement 需求 | Client Input (English) | 客戶填寫 (中文) |
| --- | --- | --- | --- |
| Machine Footprint 機台尺寸 | Must fit through standard door (32” width) | — | — |
| ATC Slot Count 刀位數量 | Minimum 4 acceptable | — | — |
| Communication Security 通訊安全性 | HTTPS required? Certificates? | No certificates needed (local only) | 不需要憑證（僅限本地運行） |
| Future Expansion 未來擴充 | Upgrade path? | Goal: run Skatescribe app directly on LinuxCNC computer (mini-PC compatible) | 目標：將 Skatescribe 應用程式直接運行於 LinuxCNC 電腦（相容迷你電腦） |